Compact constructed wetland treating combined sewer overflows in urban areas

Zone humide artificielle compacte traitant les débordements des égouts unitaires dans les zones urbaines

Storath, Juliaa; Born, Maximilian P.a; Benstoem, Frankb; Wintgens, Thomasa

a Institute of Environmental Engineering, RWTH Aachen University, Mies-van-der-Rohe-Straße 1, 52074 Aachen, Germany; b atd GmbH, Krefelder Str. 147, 52070 Aachen, Germany

RÉSUMÉ

Les zones humides artificielles compactes pour les débordements d'égouts unitaires (CSO-CW compacts) visent à offrir des performances de traitement élevées dans les zones urbaines où l'espace est limité. L'augmentation du débit par unité de surface est la principale approche pour réduire la demande en surface de filtration; cependant, une charge hydraulique plus élevée diminue le temps de séjour et nuit à l'élimination des polluants. Pour remédier à ce problème, deux stratégies d'optimisation ont été étudiées: (i) des substrats filtrants alternatifs présentant des propriétés hydrauliques et sorptives améliorées, et (ii) des floculants biodégradables (biopolymères) pour améliorer l'agglomération des particules. Six substrats ont été testés dans des colonnes à petite échelle à charge hydraulique élevée, avec et sans dosage de biopolymères, parallèlement à un criblage de 16 biopolymères à l'aide de tests en jarre. Les biopolymères à base de chitosane ont réduit la turbidité jusqu'à 80 % dans les CSO. Les essais en colonne à petite échelle ont démontré que les substrats fins et sorbants maintenaient des rendements d'élimination élevés à des débits plus importants, tandis que les matériaux grossiers montraient des améliorations substantielles avec l'ajout de biopolymères. Ces résultats indiquent que les configurations compactes CSO-CW peuvent permettre une élimination des particules et des matières organiques comparable à celle des systèmes conventionnels lorsque le choix approprié du substrat est associé à un traitement assisté par biopolymères. Cependant, même à un débit double, le CSO-CW de référence sans dosage de biopolymères n'a montré qu'une performance de traitement légèrement inférieure (<10 %) pour les paramètres clés (COD, NH4-N, TSS). Des essais pilotes semi-techniques combinant des débits accrus et un dosage de biopolymères sont actuellement évalués ; les résultats finaux sur le dosage optimal de biopolymères et les effets combinés de l'intensification hydraulique et de la floculation seront présentés lors de la conférence.

ABSTRACT

Compact constructed wetlands for combined sewer overflows (compact CSO-CWs) aim to provide high treatment performance in urban areas with limited space. Increasing the area-related flow rate is the primary approach to reducing filter surface demand; however, higher hydraulic loading decreases residence time and impairs pollutant removal. To address this issue, two optimisation strategies were investigated: (i) alternative filter substrates with improved hydraulic and sorptive properties, and (ii) biodegradable flocculants (biopolymers) to enhance particle agglomeration. Six substrates were tested in small-scale columns at elevated hydraulic loading with and without biopolymer dosing, alongside a screening of 16 biopolymers using jar tests. Chitosan-based biopolymers reduced turbidity by up to 80 % in CSO. Small-scale column tests demonstrated that fine and sorptive substrates maintained high removal efficiencies at higher throughputs, whereas coarse materials showed substantial improvements with biopolymer dosing. These results indicate that compact CSO-CW configurations can achieve particle and organic matter removal comparable to conventional systems when appropriate substrate selection is combined with biopolymer-assisted treatment. However, even at double flow rate, the reference CSO-CW without biopolymer dosing showed only slightly lower treatment performance (<10 %) for key parameters (COD, NH4-N, TSS). Semi-technical pilot tests combining increased flow rates with biopolymer dosing are currently evaluated; final results on optimal biopolymer dosage and combined effects of hydraulic intensification and flocculation will be available for presentation at the conference.

KEYWORDS

Biopolymer, Combined sewer overflow, Constructed wetland, Retention soil filter, urban runoff treatment

# Introduction

In Germany, combined sewer overflow constructed wetlands (CSO-CWs) are widely implemented nature-based solutions for mitigating emissions from otherwise untreated urban stormwater (Meyer et al., 2013; Rizzo et al., 2020; Tondera, 2019). Their treatment efficiency relies primarily on the interaction between hydraulic loading, filter substrate properties, and the retention of particulate, dissolved and oxygen-depleting substances. Conventional CSO-CWs are typically dimensioned with area-related flow rates of 0.01-0.05 L s-1 m-2, resulting in high surface area demand, often corresponding to approximately 1 % of the connected impervious catchment area (DWA, 2019; Grotehusmann et al., 2015). In dense urban environments, such spatial requirements constitute a major barrier to implementation.

To address this limitation, compact CSO-CWs are being developed to substantially reduce filter surface area while maintaining high hydraulic and treatment performance. Increasing the area-related flow rate is the only viable design lever to reduce spatial footprint; however, higher hydraulic loading inevitably shortens hydraulic residence time and can impair pollutant removal. Consequently, optimisation measures are required to compensate for these performance losses.

Two complementary strategies have been proposed for compact CSO-CWs: (i) the use of alternative filter substrates with enhanced hydraulic conductivity and sorption capacities compared with conventional sand-based filters, and (ii) the addition of biodegradable flocculation agents (biopolymers) to the inflow to promote agglomeration and improved removal of particulate contaminants. The present study investigates both approaches through laboratory-scale and semi-technical experiments, aiming to identify suitable process combinations for compact CSO-CWs configurations.

# Methods

## Screening of biopolymers

A total of 16 biopolymers based on chitosan and potato starch were evaluated in jar tests to assess their potential to enhance particle aggregation under conditions representative of CSO discharges. A simplified initial pre-screening was followed by detailed optimisation of operating parameters (optimal concentrations and mixing conditions) of selected biopolymers. Turbidity reduction was selected as the primary indicator of flocculation efficiency.

## Small-scale column tests

Six filter substrates with differing hydraulic and sorptive properties were selected for an assessment in small-scale column tests (Table 1). All substrates were tested as replicate at elevated area-related flow rates (0.1 L s-‍1 m-2) representative of compact CSO-CW operation, except for columns 1 and 2, which were operated as reference CSO-CW (0.05 L s-‍1 m-2). Influent water characteristics were adjusted to mirror typical CSO conditions by using the effluent of the primary clarifier (20 %) and the effluent of the wastewater treatment plant (80 %). Removal performance was assessed for key parameters, including turbidity, chemical oxygen demand (COD), total organic carbon (TOC), dissolved organic carbon (DOC), ammonium-nitrogen (NH4-N), ortho-phosphate (o-PO4-P), total suspended solids (TSS), and their fine fraction smaller than 63 µm (TSS63). Columns were operated without biopolymer addition to independently assess substrate effectiveness under increased hydraulic loading and with biopolymer dosing to assess biopolymer effectiveness for different substrates.

## Semi-technical pilot tests

Four semi-technical compact CSO-CWs (3.72 m² each) were operated to assess treatment performance under increased hydraulic loading. All systems were constructed according to the North Rhine-Westphalia design manual for constructed wetlands treating combined sewer overflows, comprising a 25 cm drainage layer (2/8 mm gravel), a 70 cm filter substrate layer, and a 6 cm top layer of dolomitic limestone (2/8 mm) (Grotehusmann et al., 2015). Three pilots were filled with sand (0/2 mm) as filter substrate, while the pilot was equipped with a gravel (2/8 mm) filter. Two pilots also operated in a former research project contained an additional 8 cm secondary organic layer originating from a previous full-scale CSO-CW.

Table 1: Investigated filter substrates and experimental design in the small-scale columns. C=column.

| Column | Filter substrate | Identifier | Grain size | Flow rate |
| --- | --- | --- | --- | --- |
| – | – | – | mm | L s-1 m-2 |
| C1/2 | Sand (Reference CSO-CW) | S1 | 0/2 | 0.05 |
| C3/4 | Sand | S2 | 0/2 | 0.1 |
| C5/6 | Sand/Gravel (50/50 wt.-%) | SGR | 0/8 | 0.1 |
| C7/8 | Gravel | GR | 2/8 | 0.1 |
| C9/10 | Sand/Activated Carbon (0-10 cm: 80/20 wt.-% 10-50 cm: 100/0 wt.-% 50-70 cm: 60/40 wt.-%) | SAC | 0/2.5 | 0.1 |
| C11/12 | Urban Rainshell: AA Minerals Type D | AAMin. | 0/25 | 0.1 |
| C13/14 | Sand/Zeolite (0-10 cm: 80/20 wt.-% 10-50 cm: 100/0 wt.-% 50-70 cm: 60/40 wt.-%) | SZ | 0/2.5 | 0.1 |

To investigate the applicability of compact CSO-CW operation, hydraulic loading was increased to 0.1 L s-1 m-2. To compensate for potential performance losses, a biopolymer (Heppix OT) was applied to two pilot columns using a static in-line mixer at concentrations of 2.5, 5.0, and 10.0 mg L-1. The reference CSO-CWs setup included one column operated at a conventional loading rate of 0.05 L s-1 m-2 without biopolymer, and one column operated at 0.1 L s-1 m-2 without biopolymer addition (Figure 1).

Figure 1: Flow-Sheet of the semi-technical CSO-CWs.

# Results and Discussion

## Biopolymer performance

Pre-screening tests identified chitosan-based biopolymers as most efficient for CSO treatment, achieving the highest turbidity reductions, while starch-based products showed considerably lower performance. In subsequent jar tests, chitosan products exhibited low optimal dosages and consistently high removal efficiencies, with Heppix OT and Heppix WS outperforming the starch-based alternative PolySepar SK72, which required substantially higher dosages for moderate turbidity reduction. Higher mixing speeds improved floc formation for all products without causing floc breakage, and extending flocculation duration beyond five minutes resulted in only marginal additional improvements. Overall, chitosan-based biopolymers demonstrated the most favourable combination of efficiency, stability, and operational feasibility for application in compact CSO-CWs.

## Small-scale column tests

Small-scale column tests showed high particle and organic matter removal across a range of filter substrates. Finer and sorptive media (sand, sand-zeolite, and, especially, sand-activated-carbon mixes) consistently achieved high TSS, turbidity, COD, TOC, and DOC removal at elevated hydraulic loading, with performances largely comparable to conventional CSO-CW benchmarks (Figure 2). Biopolymer dosing further improved TSS, TSS63, turbidity, COD, and TOC removal in almost all configurations, with the most substantial effect observed in coarse substrates (gravel, shells (AA Min.)), which approached the performance of finer media under flocculation-supported operation. In contrast, DOC removal remained largely unaffected by biopolymer addition and depended strongly on sorptive media such as activated carbon. Nutrient removal was mainly substrate-controlled: zeolite-containing mixes provided the highest NH4-N removal via ion exchange, while phosphorus retention was governed by mineral composition (e.g. carbonate-rich shells) and sorption capacity, with biopolymer dosing having no measurable impact on dissolved phosphate. Overall, the results demonstrate that compact CSO-CW concepts can achieve particle and bulk organic matter removal rates comparable to those of conventional systems when combined with suitable substrates and biopolymer dosing.

Figure 2: Difference (CX/Y-C1/2) in removal efficiency in a) without biopolymer dosing and b) with biopolymer dosing.

## Semi-technical pilot tests

At the time the original abstract was prepared, semi-technical experiments with biopolymer dosing were being evaluated, and no final results were yet available. The semi-technical pilot test is expected to provide insights into (i) optimal biopolymer dosage for different CSO event characteristics, (ii) the interaction between substrate properties and flocculation performance under increased hydraulic loading, and (iii) the degree to which flocculation can compensate for reduced residence time. The final results will be available for presentation at NovaTech and will refine design recommendations for compact CSO-CW configurations.

# Conclusion

The results demonstrate that compact CSO-CWs operated at doubled hydraulic loading can achieve removal efficiencies comparable to conventional systems when substrate selection and operational measures are appropriately combined. Fine-grained and sorptive media inherently ensured high retention of particulates and organic matter, while biopolymer dosing substantially enhanced the performance of coarse substrates, enabling robust particle removal across all configurations. Sorptive mechanisms remained essential for dissolved organics, with activated carbon achieving the highest COD, TOC and DOC reductions. Nutrient removal was substrate-driven, with zeolite providing the most effective NH4-N retention through ion exchange, and phosphorus removal depending on mineral composition rather than biopolymer addition. Overall, the integration of hydraulic intensification with targeted substrate selection and biopolymer dosing establishes compact CSO-CWs as a technically feasible option for space-constrained urban environments.

Acknowledgments

This project has received funding from the European Union’s Horizon Europe research and innovation programme under grant agreement No 101060428.

The authors gratefully acknowledge the valuable support provided by former colleague Leonie Wacker (formerly with ISA, RWTH Aachen University), who was primarily responsible for conducting the flocculant experiments. We further thank Leonie Borges and Kevser Gürbüz for their assistance as student research assistants, who contributed to the experimental work.

LIST OF REFERENCES

DWA (2019). Arbeitsblatt A-178 Retentionsbodenfilteranlagen (in german). Retention soil filter systems, 1st ed. Deutsche Vereinigung für Wasserwirtschaft Abwasser und Abfall, Hennef, 41 pp.

Grotehusmann, D., Uhl, M., Fuchs, S., Lambert, B. (2015). Retentionsbodenfilter-Handbuch für Planung, Bau und Betrieb (in german). Retention soil filter manual for planning, construction and operation. Ministry for Climate Protection, Environment, Agriculture, Nature Conservation and Consumer Protection of the State of North Rhine-Westphalia (MKULNV), 86 pp.

Meyer, D., Molle, P., Esser, D., Troesch, S., Masi, F., Dittmer, U. (2013). Constructed Wetlands for Combined Sewer Overflow Treatment—Comparison of German, French and Italian Approaches. Water 5 (1), 1–12. DOI: 10.3390/w5010001.

Rizzo, A., Tondera, K., Pálfy, T.G., Dittmer, U., Meyer, D., Schreiber, C., Zacharias, N., Ruppelt, J.P., Esser, D., Molle, P., Troesch, S., Masi, F. (2020). Constructed wetlands for combined sewer overflow treatment: A state-of-the-art review. Sci Total Environ 727, 138618. DOI: 10.1016/j.scitotenv.2020.138618.

Tondera, K. (2019). Evaluating the performance of constructed wetlands for the treatment of combined sewer overflows. Ecological Engineering 137, 53–59. DOI: 10.1016/j.ecoleng.2017.10.009.