Urban Drainage and Heating Cooling Networks Planning

Utiliser les égouts pour les réseaux de chaleur et de froid

Alma Schellart1, Richard Buckingham2, Stefano Giacalone3, Alemtsehay Seyoum1, Charles Rouge1, Mohamad Abdel-Aal4, Simon Tait1, David Singerton2

1 School of Mechanical, Aerospace and Civil Engineering, The University of Sheffield, UK; 2Anglian Water, UK; 3 BMA (Business Modelling Applications), UK; 4Centre for Water Systems, University of Exeter, UK; a.schellart@sheffield.ac.uk, rBuckingham@anglianwater.co.uk, SGiacalone@businessmodelling.com, a.g.seyoum@sheffield.ac.uk, c.rouge@sheffield.ac.uk, M.Abdel-Aal@exeter.ac.uk, s.tait@sheffield.ac.uk, dsingerton@anglianwater.co.uk

# RÉSUMÉ

Décarboner le chauffage et la climatisation, et moderniser les réseaux d’assainissement des eaux, sont deux défis considérables et extrêmement coûteux en infrastructure nouvelle. Utiliser l’eau des égouts comme source et puits de chaleur est une synergie qui peut diminuer les coûts associés avec ces deux défis. Pour la réaliser, les gestionnaires de réseaux de drainage et d’assainissement doivent maîtriser le régulation et connaître le contexte et les initiatives locales. En particulier, il doivent évaluer l’impact d’échanges thermiques avec des réseaux de chaleur ou de froid en divers endroits de leur réseau, pour comprendre quelle quantités de chaleur peuvent être échangées avec lui. Au-delà, ils doivent si possible mettre en place des structures de gouvernance intégrées entre les secteurs de l’eau et de l’énergie.

Le but de cet abstract et de réfléchir sur les leçons apprises lors du projet d’innovation TORCH en cours, financé par le régulateur britannique du secteur de l’eau, Ofwat. TORCH développe un outil d’aide à la décision basé sur l’intelligence artificielle pour géolocaliser les réseaux de chaleur les plus prometteurs. Ce papier explique comment les initiatives gouvernementales pour développer les réseau de chaleur ont motivé ce projet, comment la comptabilité des émissions au Royaume-Uni demeure un obstacle à leur implémentation, et comment des outils d´aide à la décision visuellement et géographiquement explicites peuvent favoriser le dialogue entre secteurs et porteurs d’enjeux.

# ABSTRACT

There is an urgent need to decarbonise heating and cooling, and district heat and cooling networks are a crucial step towards this. There is furthermore a need to upgrade ageing urban drainage systems. However, the capital costs of installing district heating and cooling networks and improving urban drainage systems are often both prohibitively high. Through innovative dual use of existing urban infrastructure as sources and sinks of thermal energy, costs may be reduced. Urban Drainage managers need to be aware of local guidance and initiatives to decarbonise heating and cooling, to be prepared to take advantage of this. This involves understanding amounts of heat that can be recovered/dumped, and potential impacts on their infrastructure. There is furthermore a need for fair investment and governance strategies integrated across the energy and infrastructure sectors.

The aim of this abstract is to reflect on lessons learned during the TORCH project, an Ofwat funded water innovation project in England, which develops an AI-assisted decision support tool to maximise heat energy recovery from urban drainage systems. The focus is on how government initiatives to encourage development of heat networks lead to the initiation of this project by a water company, how local carbon accounting is still causing challenges to implementation and roll-out of the energy recovery tool, and how visual decision support tools can help encourage dialogue between stakeholders.

# KEYWORDS

Integration energy and water sector, financing, decarbonization, district heating and cooling

# Introduction

District heating and cooling networks are a key step towards reaching the EU energy and climate targets. The EU (2023a) describes that heating and cooling accounts for 50% of its final energy consumption, and the sector is largely based on fossil fuels. As part of the European Green Deal, the revised Renewable Energy Directive came into force in November 2023 (EU/2023/2413). The directive now includes specific measures to accelerate the development of renewables in heating and cooling, as described in EU (2023b). This includes binding national targets where each EU country must increase the share of renewable energy from waste heat and cold in their district heating and cooling sector. In the UK, heat networks are also seen as a crucial tool in decarbonizing heating of the UK’s buildings. The UK’s Energy Act (UK Government, 2023) provides the powers for government to implement heat network zoning in England through regulations.

Both the revised Urban Wastewater Treatment Directive (UWWTD; EU 2024) and the UK Environment Act (UK Government, 2021) strongly encourage improving stormwater management systems, which is expected to bring considerable costs.

Loveridge et al. (2022) describe how capital costs of ground source heat recovery and district heating technology are often high. However, these are anticipated to be reduced by innovative approaches using existing infrastructure sources and stores of thermal energy. Barriers to such innovative approaches include lack of knowledge about the actual net amount of recoverable energy, impacts on the primary function of any buried infrastructure, as well as the need for new investment and governance strategies integrated across the energy and infrastructure sectors.

Some examples of linking urban drainage systems to heating and cooling systems already exist, yet these are mainly focussed on single users and combined or foul sewer systems, for example at the water utility Aquafin’s own offices, Ingenium (2021). The practise is not widespread, although initiatives are emerging such to link heat off takers and urban drainage operators, such as the Aquamarkt in Flanders, Belgium (AquaMarkt, 2024) and the Toronto Wastewater Energy Map (City of Toronto, 2025). Further opportunities to utilise storm drainage systems include using shallow ground source heat pump systems for domestic heating to achieve a better performance when linked to stormwater infiltration systems (Ali et al., 2017). The infiltrating stormwater locally increases soil moisture, which in turn enhances the heat transfer between the soil and the ground heat exchanger.

In a warmer future, a worldwide rise in cooling needs is expected to cause an increasing strain on power network during peak times (IEA, 2025). Utilising urban drainage infrastructure for heat energy dumping is a means to both alleviate this strain and to manage urban heat island effects.

Hence there is a growing opportunity for cost efficiency through double use of urban drainage systems as sources and sinks for thermal energy.

# ReflECTIONS from the TORCH project

The UK Government’s Department for Energy Security and Net Zero (DESNZ) has designated zones in urban areas where heat networks are expected to offer the lowest-cost solution for decarbonising heat (DESNZ, 2024). With this initiative, DESNZ aims to provide local communities with planning tools to accelerate the development of heat networks, and to ensure that more homes and businesses can access greener, cheaper heat. The city of Peterborough in England, which has approximately 218,000 inhabitants, is one of these designated heat network zones. Peterborough is served by the water company Anglian Water Services Ltd and contains a mixture of separated and combined sewer systems.

DESNZ also initiated the development a guidance note for use by wastewater companies and heat networks developers. It contains ‘rules of thumb’ for establishing Exclusion Zones around sewer heat recovery projects, to avoid detrimental competition for the heat resource (WSP, 2025).

Both documents (DESNZ, 2024 and WSP, 2025) caused considerable concern with Anglian Water, as they felt the Exclusion Zones screening tool does not adequately inform on the risk of heat recovery to the functioning of their assets. They are also concerned that the designation of the heat network zone in Peterborough would lead to heat network providers being empowered to extract heat energy from their sewer systems without adequate consideration of the operational risks to the water utility. The water company does not want to be seen as hampering local decarbonization efforts, but equally it does not want to get blamed if the performance of the sewer system were to deteriorate.

To resolve this quandary, Anglian Water therefore took the initiative for the TORCH project (Tool for Optimising decisions for the Recovery of sewer Catchment Heat). This project is funded by the England and Wales water sector regulator, Ofwat, through its dedicated innovation fund. The TORCH project aims to develop an AI-assisted decision support tool to maximise heat energy recovery from sewers and wastewater treatment plants by identifying potential heat source locations, heat energy recovery technology, practicality barriers and transmission routes. The need for AI-assisted decision support tools is primarily related to the current absence of a consistent way for water companies to respond to the increasing number of queries from energy companies and local administration regarding the potential to access sewer heat.

This tool will be linked to the DecisioTM Platform (BMA, 2025) to provide an effective and visual way for the different stakeholders to have an objective way to assess the risks and opportunities related to use of waste heat from sewers to power district heat networks (Figure 1).

Figure 1. Integrating Waste Heat Supply and Demand Matching into the DecisioTM Platform.

Heat extraction from sewers will deliver carbon savings to the users of this heat, and the TORCH project seeks to quantify these. These carbon savings will depend upon the energy source displaced by sewer heat, as well as on the pace of decarbonisation of the power grid. Currently, the UK government quantifies heat network carbon benefits against the benchmark of installing an air source heat pump, rather than against the benchmark of keeping gas heating systems. This approach minimizes the carbon benefits one can claim heat network investments provide, and it also does not account for the reduction in the need to strengthen the electricity transmission grid that may be achieved through accessing higher grade heat sources. What is more, whilst the water company would facilitate the use of this heat, there is no carbon accounting mechanism in place to attribute a proportion of these carbon savings to water utilities. Work is underway to develop such a mechanism, because it would provide an increased incentive for water companies to facilitate access to their assets.

# CONCLUSION

The integration of heating and cooling networks with urban drainage networks can generate financial benefits to both sectors, however, better local legislation and regulation would be needed to enable this.

For both the water and the energy sector to gain financial benefit from linking heating and cooling networks to urban drainage systems, the governance around carbon accounting needs to be carefully reviewed, and fair distribution of carbon credits is needed.

Urban drainage managers and water utilities need to be aware of local heating and cooling networks initiatives. More of these are expected to be initiated, considering the binding EU targets for increasing the share of renewable energy and the key interest in heating and cooling networks. As district heating and cooling networks are key in the transition to low carbon heating and cooling, water utilities throughout Europe are expected to face more requests from energy providers.

The use of a multi-stakeholders planning platform, such as developed in the TORCH project, can stimulate dialogue between interested parties and reduce the complexity in initiating innovative projects that link district heat and cooling networks to urban drainage assets.

ACKNOWLEDGEMENT

The TORCH project is funded by the Ofwat Innovation Fund  (5th Water Breakthrough Challenge).

# LIST OF REFERENCES

Ali., A., Mohamed, M., Abdel-Aal, M., Schellart, A., Tait, S. (2017). Analysis of ground-source heat pumps in north-of-England homes. Proc. of the Inst. of Civil Engineers, Energy, Vol 170, Issue EN3, pp 103–115 http://dx.doi.org/10.1680/jener.15.00022

Aquafin (2024). AquaMarkt. https://www.aquamarkt.be/pages/warmte [Accessed: 2nd Dec. 2025].

BMA (2025). About DecisioTM. https://www.businessmodelling.com/about/ [Accessed: 2nd Dec. 2025].

City of Toronto (2025). Wastewater Energy Projects. https://www.toronto.ca/services-payments/water-environment/net-zero-homes-buildings/wastewater-energy-projects/ [Accessed: 2nd Dec. 2025].

DESNZ (2024), Department for Energy Security and Net Zero, Heat network zoning, 2024. https://www.gov.uk/government/collections/heat-network-zoning. [Accessed: 7th Nov. 2025].

EU (2023a) Renewable heating and cooling pathways – Towards full decarbonisation by 2050 – Final report, Publications Office of the European Union, 2023, https://data.europa.eu/doi/10.2833/036342

EU (2023b). Communication from the Commission – Guidance on heating and cooling aspects in Articles 15a, 22a, 23 and 24 of Directive (EU) 2018/2001 on the promotion of the use of energy from renewable sources as amended by Directive (EU) 2023/2413. ELI: http://data.europa.eu/eli/C/2025/2238/oj ISSN 1977-091X (electronic edition)

EU (2024) Directive (EU) 2024/3019 of the European Parliament and of the Council of 27 November 2024 Concerning Urban Wastewater Treatment (Recast). Available at: https://eur-lex.europa.eu/eli/dir/2024/3019/oj/eng [Accessed 10th June 2025]

IEA (2025). Staying cool without overheating the energy system. https://www.iea.org/commentaries/staying-cool-without-overheating-the-energy-system [Accessed: 2nd Dec. 2025].

Ingenium (2021). Aquafin pioneers riothermal energy on own site in Aalst. https://en.ingenium.be/nieuws/aquafin-pioniert-met-riothermie-op-eigen-site-in-aalst [Accessed 2nd Dec. 2025].

Loveridge F, Schellart A, Rees S, Stirling R, Taborda D, Tait S, Alibardi L, Biscontin G, Shepley P, Shafagh I , Shepherd W, Yildiz A, Jefferson B (2022). The potential for heat recovery and thermal energy storage in the UK using buried infrastructure. Proceedings of the Institution of Civil Engineers - Smart Infrastructure and Construction, 175(1), 10-26. UK Government (2021) Environment Act (2021). Available at: https://www.legislation.gov.uk/ukpga/2021/30/contents/enacted [Accessed: 2nd Dec. 2025].

UK Government (2023). Energy Act 2023, https://www.legislation.gov.uk/ukpga/2023/52/introduction/enacted [Accessed 26th Nov. 2025].

WSP (2025). Department for Energy Security and Net Zero, Sewer Heat Recovery – Exclusion Zone Guidance Methodology, 2025. https://www.gov.uk/government/publications/sewer-heat-recovery-exclusion-zone-guidance-methodology. [Accessed: 11th Nov. 2025].