Localized Sediment Resuspension Model: Evaluating Transferability and Extending Resuspension Dynamics with PSD Information

Localized Sediment Resuspension Model : Évaluation de la transférabilité et extension des dynamiques de remise en suspension avec les informations PSD

Vasileios Chrysochoidis1,2*, Dawei Yu3, Peter Steen Mikkelsen1, Luca Vezzaro1

1 Department of Environmental Engineering, Technical University of Denmark

2 Sino-Danish Center for Education and Research (SDC)

3Research Center for Eco-Environmental Sciences, Chinese Academy of Science, Beijing, China

# RÉSUMÉ

Les déversements d’eaux usées unitaires (CSO) présentent une dynamique polluante fortement variable, les pics de matières en suspension (TSS) étant souvent liés à la remise en suspension de sédiments accumulés dans le réseau. La prévision de ces pics reste difficile en raison de l’hétérogénéité spatiale marquée des processus sédimentaires et de la disponibilité limitée de données de qualité d’eau à haute résolution. Le modèle Localized Sediment Resuspension (LSR) répond à ces défis en représentant l’accumulation et la remise en suspension des sédiments au moyen de dynamiques locales ciblant des zones spécifiques du réseau. Cette étude évalue la transférabilité du modèle LSR en l’appliquant à un nouveau bassin d’assainissement unitaires à Dresde, en Allemagne, et examine si ses principes fondamentaux restent valables sous des conditions hydrauliques et structurelles différentes. Le modèle est ensuite étendu par l’introduction de classes de sédiments issues de distributions granulométriques (PSD), permettant aux seuils d’érosion et aux cinétiques de refléter les propriétés intrinsèques des matériaux. Les résultats montrent que le modèle LSR parvient à reproduire les pics de TSS liés à la remise en suspension dans ce nouveau bassin, confirmant ainsi sa transférabilité. L’introduction de classes de sédiments a révélé des comportements de mobilisation spécifiques aux fractions grossières, moyennes, fines et très fines, soulignant l’importance de l’hétérogénéité des matériaux. Enfin, les analyses de scénarios montrent que la structure du modèle et la composition granulométrique influencent la prédiction des TSS. Ensemble, ces résultats renforcent la base physique et la généralité du modèle LSR pour la modélisation de la dynamique des polluants en CSO.

# ABSTRACT

Combined sewer overflows (CSOs) exhibit highly variable pollutant dynamics, with Total Suspended Solids (TSS) peaks often driven by the resuspension of sediments accumulated within the network. Predicting these peaks remains difficult due to strong spatial heterogeneity in sediment processes and the limited availability of high-resolution water-quality data. The Localized Sediment Resuspension (LSR) model addresses these challenges by representing sediment accumulation and resuspension through localized hotspot dynamics. This study evaluates the transferability of the LSR model by applying it to a new combined sewer catchment in Dresden, Germany, and examines whether its core principles remain valid under different hydraulic and network conditions. The model is further extended by introducing sediment classes derived from particle size distribution (PSD) information, allowing erosion thresholds and kinetics to reflect intrinsic sediment properties. Results show that the LSR model is able to reproduce resuspension-driven TSS peaks in the new catchment, confirming its applicability beyond the original system. Introducing sediment classes revealed additional mobilization behaviour across coarse, medium, fine, and very fine fractions, highlighting the role of material heterogeneity. Scenario analyses further demonstrated that both model structure and PSD composition influence TSS predictions. Together, these results strengthen the physical basis and generality of the LSR framework for modelling CSO pollutant dynamics.

# KEYWORDS

Urban drainage systems, Sewer Network Modeling, Pollution-based real-time control (PB-RTC), Peak-Weighted Dynamic Time Warping (PW-DTW), Spatial Heterogeneity.

# Introduction

Combined sewer overflows (CSOs) continue to represent a major challenge for urban water management, as they discharge a mixture of untreated wastewater and stormwater during wet-weather events and often exhibit highly variable pollutant dynamics (Perry et al., 2024). Total Suspended Solids (TSS) are particularly important in this context because they act as carriers for a wide range of contaminants and their concentrations can rise sharply when previously deposited sediments are mobilized (Gasperi et al., 2008). However, predicting TSS peaks remains difficult due to the inherent spatial and temporal heterogeneity of sediment accumulation and resuspension processes, as well as the limited availability of high-resolution water-quality data (Gao et al., 2023). Traditional modelling approaches struggle with these dynamics because they generally assume spatially uniform sediment behaviour and rely on simplified accumulation–wash-off formulations (Chrysochoidis et al., 2025). As a result, they often fail to represent the localized erosion processes that govern event TSS peaks.

To address these challenges, the Localized Sediment Resuspension (LSR) model is built on the principle that sediment accumulation and resuspension are highly localized and shaped by strong spatial heterogeneity throughout the sewer network. By representing these heterogeneous processes through localized resuspension dynamics, the LSR framework captures the event-dependent mobilization patterns that drive TSS peaks.

While the LSR model captures the essential role of localized processes, applying it more broadly requires assessing its performance under different system conditions. In this study, the LSR model is therefore evaluated in a new catchment to examine whether its underlying principles remain valid when hydraulic characteristics, network configurations, and monitoring conditions differ from those of the original system. In addition, the initial formulation treated accumulated sediment as a uniform material, even though particle size distributions (PSD) vary substantially across locations and events and can strongly influence erosion thresholds and resuspension behaviour.

In this study, we examine the transferability of the LSR approach by applying it to a new catchment and assessing whether the principles of localized sediment accumulation and resuspension remain valid under different hydraulic and network conditions. We also extend the model to include class-specific erosion behaviour derived from PSD information, allowing erosion thresholds and kinetics to reflect intrinsic sediment properties while preserving the spatially heterogeneous structure of the LSR model. Together, these developments aim to strengthen the generality and physical basis of the LSR model principles and improve its ability to describe resuspension-driven TSS dynamic.

# Methods

## Governing Equations

The LSR model describes sediment mobilization by explicitly representing localized accumulation and resuspension locations (referred to as hotspots) across the sewer network. Using outputs from a detailed hydraulic model, pipe stretches exhibiting near-zero dry-weather velocities are identified as potential accumulation zones. For each hotspot j, the evolution of accumulated mass is described through a discrete mass balance:

| Mjt=Mjt-1-AERjtΔt | Equation 1 |
| --- | --- |

where Mj(t) [g] is the sediment mass and AERj(t) [g/s] is the actual erosion rate at timestep t. The initial mass Mj(0) is estimated through calibration within predefined bounds derived from historical event data. Although the antecedent dry period is not explicitly modelled, its influence is implicitly captured in this parameter. Resuspension occurs only when the local bed shear stress τj(t), obtained from the hydraulic simulation, exceeds the critical shear stress τcrit,j(t). The resuspended material is transported downstream using a Lagrangian shift based on hotspot-specific travel times, and the total TSS at the measurement location is obtained by superimposing all hotspot contributions onto the hydraulic model’s initial (“baseline”) TSS predictions.

In this study, sediment heterogeneity was incorporated in the LSR model by introducing sediment classes derived from PSD information collected in a case study of combined sewer networks in China. Four classes were defined based on observed distributions: coarse (>0.85 mm, 27%), medium (0.25–0.85 mm, 30%), fine (0.149–0.25 mm, 34%), and very fine (<0.149 mm, 9%). In this extended formulation, all governing equations retain their original structure but are evaluated separately for each sediment class s. The mass balance becomes:

Mj,st=Mj,st-1-AERj,stΔt			    Equation 2

and the resuspension flux is updated accordingly:

JLSRt=s=1Nclassesj=1NhotspotsAERj(t-tj,ev*)   			Equation 3

allowing erosion thresholds and kinetics to reflect intrinsic material properties.

This study evaluates these developments using high-resolution hydraulic and TSS data from a combined sewer catchment in Dresden, Germany.

Model calibration was performed independently for each wet-weather event using the Particle Swarm Optimization (PSO) algorithm, applied to 16 calibration events where the observations indicated potential for resuspension. Model performance was assessed using the Peak-Weighted Dynamic Time Warping (PW-DTW) metric, a novel extension of DTW developed to emphasize the accurate reproduction of resuspension-driven TSS peaks. The calibrated parameters were then evaluated on 10 independent verification events, also selected based on observed indications of resuspension, to assess model robustness and transferability.

Stochasticity is introduced in two parts of the LSR model. First, uncertainty in erosion thresholds is represented by sampling values of τcrit,j from calibrated distributions for each event. Second, hotspot activation is treated probabilistically, acknowledging that not all hotspots contribute during every rainfall event.

A model structure scenario analysis was performed to evaluate how increasing parameter heterogeneity influences the representation of resuspension processes. A separate PSD scenario analysis assessed the model’s sensitivity to sediment composition by testing alternative coarse-, medium-, and fine and very fine-dominated particle size distributions.

# Results and Discussion

Across the new case study, the hydraulic analysis identified 10 sediment accumulation hotspots distributed throughout the system. The performance of the LSR model on two example verification events is shown in Figure 1. In these events, the model successfully captured the presence of the TSS peaks, reflecting the dominant pollutant dynamics driven by resuspension. Some discrepancies remained in the exact timing and magnitude of the peaks, yet the general event-scale behaviour was reproduced with reasonable agreement. These results indicate that the hotspot-driven formulation of the LSR model is transferable and capable of describing resuspension-driven pollutant responses in sewer systems beyond the original case study.

Figure 1. Observed (black), baseline predicted (blue), and LSR predicted (red) TSS profiles for events 14 (left) and 30 (right).

Introducing sediment classes revealed additional dynamics in the behaviour of the LSR model Figure 2. By evaluating erosion thresholds and kinetics separately for coarse, medium, fine, and very fine fractions, the extended formulation produced a more differentiated distribution of resuspension fluxes and highlighted distinct mobilization patterns across sediment classes. Event-level analyses showed clear separation in how different fractions responded to hydraulic forcing, providing a more detailed representation of peak formation mechanisms. These findings indicate that sediment heterogeneity, and particularly differences in erodibility across PSD fractions, plays an important role in shaping TSS responses during wet-weather events.

Figure 2. Observed (black), baseline predicted (blue), and LSR predicted TSS profiles decomposed into four sediment classes (coarse: orange, medium: green, fine: purple, very fine: cyan) for events 14 (left) and 30 (right).

Moreover, evaluation of different levels of model-structure complexity showed that the formulation in which each sediment class is assigned its own erosion threshold and erosion kinetics consistently ranked among the best-performing structures across the analyzed events. Formulations that assumed a single uniform sediment type were able to reproduce some events reasonably well, but their performance was more variable, whereas the class-specific structure provided a more stable description of event-scale behaviour. This pattern indicates that incorporating both spatial heterogeneity (hotspots) and material heterogeneity (sediment classes with distinct erodibility) yields a more robust representation of resuspension processes.

Finally, the PSD scenario analysis highlighted the sensitivity of the LSR model to sediment composition. Testing four plausible PSD splits revealed distinct mobilization behaviour, with coarse-dominated scenarios producing sharper and earlier peaks, whereas fine-dominated scenarios generated smoother and more prolonged responses. The reference PSD scenario provided the most balanced agreement across events, while the more extreme scenarios demonstrated how deviations in sediment composition can propagate into event-scale TSS predictions. These differences indicate that the model responds strongly to how sediment mass is distributed across particle size classes within hotspot locations.

This sensitivity underscores the importance of case-specific PSD information where available, but also shows that scenario testing can offer insight into likely catchment conditions when such measurements are lacking. In the present study, the results point to a sediment composition that is predominantly medium- to fine-dominated, with coarse fractions contributing only marginally. Such distributions are typical in urban environments, where road dust and traffic-related particles (e.g., tyre and brake wear, pavement abrasion) represent major sources, and may also reflect in-sewer processes that favour the retention of coarse particles upstream (e.g., in gully pots) while allowing medium and fine fractions to accumulate and later resuspend. At the same time, the relatively strong performance of the reference PSD, despite being imported from another system, indicates that external distributions can approximate local conditions in some cases. Nonetheless, relying on a single assumed PSD remains uncertain, and exploring a range of plausible compositions provides a more robust basis for representing system behaviour in the absence of local measurements.

# Conclusion

This study demonstrated that the principles of the LSR model remain valid when applied to a new catchment with different hydraulic and network conditions. Incorporating sediment classes revealed additional mobilisation dynamics and provided a more physically grounded representation of resuspension behaviour. The combined assessment of model transferability, structural complexity, and PSD sensitivity highlights the importance of accounting for both spatial and material heterogeneity when predicting TSS peaks in combined sewer systems.

LIST OF REFERENCES

Chrysochoidis, V., Gruber, G., Hofer, T., Mikkelsen, P. S., & Vezzaro, L. (2025). Rethinking modelling of particulate pollutants in combined sewer overflows (CSOs): A focus on model structure. Journal of Hydrology, 659, 133239. https://doi.org/https://doi.org/10.1016/j.jhydrol.2025.133239

Gao, Y., Shi, X., Jin, X., Wang, X. C., & Jin, P. (2023). A critical review of wastewater quality variation and in-sewer processes during conveyance in sewer systems. Water Research, 228, 119398. https://doi.org/https://doi.org/10.1016/j.watres.2022.119398

Gasperi, J., Garnaud, S., Rocher, V., & Moilleron, R. (2008). Priority pollutants in wastewater and combined sewer overflow. The Science of the Total Environment, 407, 263–272. https://doi.org/10.1016/j.scitotenv.2008.08.015

Perry, W. B., Ahmadian, R., Munday, M., Jones, O., Ormerod, S. J., & Durance, I. (2024). Addressing the challenges of combined sewer overflows. Environmental Pollution, 343, 123225. https://doi.org/https://doi.org/10.1016/j.envpol.2023.123225