Automated Generation of rules for Real Time Control of urban drainage systems

Génération automatisée de règles pour le contrôle en temps réel des réseaux d'égouts

Manfred Schütze1, Jens Alex1

1ifak e. V. Magdeburg, Germany (manfred.schuetze@ifak.eu, jens.alex@ifak.eu)

RÉSUMÉ

Cette contribution présente une méthode pour la génération automatisée d'un algorithme de contrôle en temps réel (RTC) pour les systèmes de drainage urbain. Après une analyse topologique automatisée du système de drainage, un ensemble de règles Fuzzy est créé spécifiquement pour le système donné. L'application réussie de cette méthode au système de référence Astlingen sert de preuve de concept de cette méthodologie. Cette méthode présente un fort potentiel, car elle permet d'évaluer le potentiel RTC d'un réseau donné en établissant rapidement un algorithme RTC, fondé sur des concepts de contrôle largement appliqués.

ABSTRACT

This contribution presents a method for the automated generation of real time control (RTC) algorithms for urban drainage systems. After an automated topological analysis of the drainage system, a Fuzzy rule set is created specifically for the given system. Successful application of this method to the Astlingen benchmark system serves as a proof of concept of this methodology. This method shows high potential as it allows to assess RTC potential of a given network by quickly establishing a rule-based RTC algorithm based on widely applied control concepts.

KEYWORDS

Automated procedure, control algorithm, Fuzzy rules, Real time control (RTC), urban drainage system

# MOtivation

Real time control (RTC) can be a cost-efficient measure to improve the performance of urban drainage system (see, for example, Dirckx, 2014). Recent years have seen significant development and application work to make RTC more accessible to practical application: This included simulator development to make model-based predictive control accessible from within the platform used for simulation (Schütze and Alex, 2022), it included the setup of generalised controller concepts, easily configurable according to network layout (Pabst et al., 2011), to name just a few.

However, despite its optimality, there is some reluctance with regard to apply model-based predictive control in practice, as it is often perceived as “non-explainable” or black-box type of RTC. On the other hand, despite the generalised concept of distributed hierarchical controllers represented an early example of the modern “Internet of Things” technology. It also has been successfully implemented in practice, for example for RTC in the city of Hildesheim (Pabst et al., 2011). However, it has not found widespread application, probably due to a perception of it being “too complicated”.

The present contribution, hence, combines and further develops several lines of research, leading to a first-time ever method of creating a (rule-based) real-time control strategy, specific to the given network, its structure and its dimensions, on the push of a button. The contribution outlines the fundamental concepts, illustrates its application on the Astlingen benchmark example as a proof of concept and, finally, provides an outlook to the far-reaching implications of this development.

# Underlying RTC Concept

As the underlying concept for the rule-based RTC to be developed in a generalised way, the generalised concept of hierarchical, distributed controllers is applied. Detailed descriptions of this concept can be found elsewhere (Alex et al., 2008, Pabst et al., 2011), in a nutshell it can be described as follows: Controllable storage volumes in the system (e.g. storage tanks or sections of sewers with significant in-pipe storage) are controlled by controllers, which are coordinated by supervisory controllers. Supervisory controllers ensure balancing (i.e. aiming at similar) filling degrees of neighbouring sections of the network by defining set points of filling degrees and by assigning flow capacities within the given restrictions of available capacities). The configuration of controllers follows the structure of controllable volumes in the system, be it in parallel branches or/and in sequential sections of the network. The Equal-Filling Degree algorithm as defined by Dirckx et al. (2014) resembles a simplified version of this concept.

Whilst the original concept of generalised controllers requires not filling degree (i.e. water level) sensor information, but also flow information at certain locations in the network, which might not be available, a more parsimonious version is employed here and represented by a set of Fuzzy rules. Fuzzy rules are a concept widely applied in RTC projects (e.g. Fuchs and Beeneken, 2005) and are believed to be more easily understandable than other RTC concepts.

The concepts underlying the controllers responsible for coordinating storage capacities (by ensuring similar filling degrees in neighbouring sections, thus avoiding situations where there is overflow in one part of the system whilst its neighbouring section still has ample free capacities) are expressed by rules comparing filling degrees of neighbouring sections (using as auxiliary variables calculated differences of filling degrees of neighbouring storage volumes within the system). The analysis of the network uses routines by Schütze et al. (2024). This leads to an overall system of 3*n rules (n being the number of controllable tanks) and n*(n-1)/2 auxiliary variables; as sensor information n filling degrees (obtainable form water levels using geometric volume-depth relationships) are required. This is a rather modest size of rule set; as rule sets are created as XML files, these are readable and editable and can be, if so required, extended by the user according to any particular needs).

# Application

## Simulation and control environment

As test environment for simulation and RTC application, the simulation system Simba# (Version 7) (ifak, 2025) is used here. Simba# allows fully hydrodynamic, but also hydrologic, simulation of sewer systems, including fully flexible options for pollutant transport and biochemical pollutant transformation processes. Furthermore, it provides a large set of options to model control systems, making it to a unique simulator allowing all three forms of RTC (rule-based (conventional and fuzzy rules), generalised-controller-based; model-based predictive) control options to be modelled. A dedicated Fuzzy rule interpreter module has been developed within Simba# is used as rule interpreter in the examples described here. The rule sets created are XML-files, thus easily readable.

## The Astlingen case study

The Astlingen benchmark example has been defined by the DWA working group on Real time control; it is fully described by DWA (2018); furthermore, it has been discussed, also in English language, in numerous other publications (e.g. Schütze et al., 2018). It resembles a sewer system (mainly combined, yet with some separate system parts) in a set-up typical for Central European conditions. Figure # provides a sketch of the storage tanks and of the related control task, whilst Table 1 provides the corresponding key numbers. Rainfall data with spatial distribution (4 raingages) is provided as part of the benchmark example.

The RTC task consists in determining, at every control time step (assumed to be 5’ here), the optimum setting for the controllable throttle devices of Tanks 2, Tank 3, Tank 4 and Tank 6 within the given ranges [qmin, qmax].

Figure 1: System sketch Astlingen (Source: DWA, 2018)

Table 1: Main RTC parameters of Astlingen system

## Results

Feeding the procedure to create a Fuzzy-based rule set with information about the urban drainage system (network model and data defining the RTC task as those shown above) creates a set of Fuzzy rules (see Figure 2).

Figure 2: Fuzzy rules automatically created for the Astlingen example (Excerpt)

Table 2 provides the result of testing (applying these rules) for a given time series input of one year of continuous (spatially distributed) rainfall data and compares the result obtained to those obtained with other RTC methods as reported in the literature (DWA, 2018, Schütze and Alex, 2022). The FuzzyRule set, despite its simplicity, performs quite well in comparison with the other approaches.

Table 2: Results of different RTC algorithm for the Astlingen example (one year continuous rainfall data input)

# Conclusions / Outlook

Various results of earlier research (including generalised controller concepts, automated analysis and simplification of arbitrary hydrodynamic sewer networks) have been combined, further developed and integrated in a convenient, state-of-the-art simulation environment. Application on the Astlingen benchmark example served as a successful proof of concept and now opens up the way to practical applications, which also will be reported on in the conference presentation.

After some extensions for non-dendritic networks, further methodological work will include the integration into an automated procedure to test all 2n combinations of using or not any selection out of n tanks for control. This will substitute the Central Basin Approach (Einfalt and Stölting, 2002), which, despite its strong assumptions, is still recommended so far for this task of identification of the best selection of tanks to be controlled, by a method providing an RTC algorithm for the selection of tanks identified. In general, any analysis of sewer systems can now easily include a check for its RTC potential with little effort.

LIST OF REFERENCES

Alex, J., Schütze, M., Ogurek, M., Jumar, U. (2008): Systematic Design of Distributed Controllers for Sewer Networks; IFAC World Congress, Seoul 2008

Dirckx, G., Van Assel, J., Weemaes, M. (2014): Real Time Control From Desk Study to Full Implementation, 13th International Conference on Urban Drainage, Kuching, Malaysia, 7 - 12 September 2014

DWA (2018): (2018): DWA-Themen: Abflusssteuerung in Kanalnetzen - Anwendungsbeispiel. ISBN 978-3-88721-697-9, DWA-Arbeitsgruppe ES-2.4 "Integrale Abflusssteuerung"; Hennef/Sieg, November 2018, 49 pp.

Einfalt T., Stölting B. (2002). Real-Time Control for two communities - technical and administrational aspects. 9th Int. Conference on Urban Drainage; Portland; September 2002

Fuchs, L., Beeneken, T. (2005): Development and implementation of a real-time control strategy for the sewer system of the city of Vienna, Water Science and Technology, 52. 5. 187–194

ifak (2025): Simba#, Version 7, Manual. Institut für Automation und Kommunikation e. V., Magdeburg

Pabst, M., Alex, J., Beier, M., Niclas, C., Ogurek, M., Peikert, D., Schütze, M. (2011): ADESBA - A new general global control system applied to the Hildesheim sewage system. 12th International Conference on Urban Drainage. Porto Alegre, 11. – 16.09.2011

Schütze, M.; Alex, J. (2003): Advanced pumping station control for RTC of sewer systems; in: Cabrera, E; Cabrera Jr. (eds.): Pumps, Electromechanical Devices and Systems Applied to Urban Water Management; Vol. 2; A.A. Balkema, Lisse, ISBN 90-5809-560-6; 887-894

Schütze, M., Lange, M., Pabst, M., Haas, U. (2018): Astlingen – a Benchmark for Real-Control Control (RTC); Wat. Sci. Tech., Water Sci Technol (2018) 2017 (2): 552-560, DOI: 10.2166/wst.2018.172

Schütze, M., Alex, J. (2022):  Steuerung und Regelung von Abwasserkanalnetzen – ein praxisgerechter Ansatz zur modellprädiktiven Regelung; Automation 2022; Baden-Baden, 28./29.06.2022; VDI-Berichte 2399, VDI-Verlag Düsseldorf GmbH; ISBN 978-3-18-102399-0; 187 – 204

Schütze, M., de Matos, K., Schmidt, T., Alex, J. (2024): Scalable model simplification for hydrodynamic sewer system models, 16th Int. Conf. on Urban Drainage, Delft, 09. – 14.06.2024

Schütze, M., Hübner, C., De Toffol, S., Kraus, B., Maßmann, S., Pabst, M., Alex, J. (2025): Automated set-up of a Digital Twin Model of the sewer system of Hannover/Germany; Conference “Urban Drainage Modelling” (UDM 2025) – Innsbruck, 15. – 19.09.2025