A new dynamic model for small-diameter suspended solids in sewer systems, supported by an extensive measurement campaign

Un nouveau modèle dynamique pour les solides en suspension de petit diamètre dans les réseaux d'égouts, fondé par une vaste campagne de mesures

Manfred Schütze1, Volker Pick2, Martin Oldenburg2

1ifak e. V. Magdeburg, Germany (manfred.schuetze@ifak.eu)

2Technische Hochschule Ostwestfalen-Lippe, Höxter, Germany (volker.pick@th-owl.de, martin.oldenburg@th-owl.de)

RÉSUMÉ

Cet article présente une approche de modélisation pour la modélisation de la production, du lessivage et de la réduction des matières en suspension totales de petit diamètre (en allemand : Abfiltrierbare Stoffe AFS63 et AFScoarse) dans les réseaux d'égouts. Le développement du modèle a été soutenu par des campagnes de mesure approfondies et à haute résolution des eaux de ruissellement routier sur trois sites en Allemagne, couvrant un total de 99 événements.

Une caractéristique novatrice de l'approche de modélisation développée ici est la définition d'une valeur seuil q1 [m/h] indépendante du bassin versant, au-dessus de laquelle se produit l'entrée d'AFS. Le modèle de nettoyage applique de manière dynamique les formules d'efficacité connues dans la littérature en déterminant la charge superficielle actuelle pour chaque intervalle de temps sur la base du débit actuel et en l'utilisant pour déterminer l'efficacité d'élimination dans l'intervalle de temps actuel. À l'aide de facteurs d'étalonnage (sans unité) (αAFS63 et αAFScoarse), il est également possible de prendre en compte les cas dans lesquels les efficacités d'élimination réelles s'écartent considérablement dans la pratique de celles spécifiées dans la littérature.

ABSTRACT

This paper presents a modelling approach for modelling of generation, washoff and reduction of small-diameter Total Suspended Solids (in German: Abfiltrierbare Stoffe AFS63 and AFScoarse) in sewer systems. Model development has been supported by extensive, high resolution measurement campaigns of road runoff at 3 sites in Germany, covering a total of 99 events.

One novel feature of the modelling approach developed here is the definition of an catchment-independent threshold value q1 [m/h], above which AFS entry occurs. The cleaning model dynamically applies the efficiency formulas known from the literature by determining the current surface loading for each time step based on the current discharge and using this to determine the removal efficiency in the current time step. Using (unitless) calibration factors (αAFS63 and αAFScoarse), cases can also be taken into account in which the actual removal efficiencies deviates in practice significantly from those specified in the literature.

KEYWORDS

AFS63, measurement campaign, model, small-diameter TSS

# MOtivation

In recent years, the consideration of pollution of rainfall-runoff, in particular from road surfaces, plays an increasing role in urban hydrology. Road runoff contains a mix of pollutants, originating, for example, from tyres and breaks, heavy metals, petrol, de-icing agents and others. The relevant guideline document in Germany DWA-A-102-2 (DWA-BWK, 2020) therefore considers the surrogate parameter AFS63 as a surrogate to describe such pollution. AFS63 is hereby understood as small-diameter (< 63 μm) of filtrable Total Suspended Solids. Larger filtrable TSS are denoted here as AFScoarse. For the assessment of impacts of runoff to surface water bodes, the A102 guidelines suggests the calculation of pollutant loads based on fictitious loads per surface area.

Within simulation models, the AFS63 parameter usually is considered assuming constant concentrations in rainfall runoff. Models often assume a build-up during dry periods (accumulation), which is washed off during rainfall events (see, for example, SWMM, Rossman and Simon, 2022).  Vinck et al. (2023) have extended this approach also to include selected heavy metals and also TSS. Some authors, e. g. Fettig et al. (2020), observed from their sampling campaigns that, instead of a complete was-off, solids might also be stored on the road surface, resulting in AFS63 loads being washed off even after intense rain events.

Therefore, a modelling approach has been setup, considering this effect. This model has been developed, tested and validated based on intensive in-situ AFS63 and AFScoarse sampling campagins, covering 99 events at three different sites with different characteristics.

# MEASUREMENT CAMPAINGS

## Case studies

Extensive measurement campaigns have been set up at three different sites in the cities of Paderborn and Warendorf (both cities located in the Westphalia region of Germany). Each of these sampling campaigns included several months of high-resolution sampling of flows and concentrations of AFS63 and AFScoarse in inflow and outflow of devices for rainwater treatment (sedimentation). A range of different characteristics is covered by the selected sampling sites:

- Residential area in a rural setting with through traffic

- Combined area (residences, small commerce)

- Traffic area (large intersection of main roads)

The calculation of removal efficiencies was based on loads and flow-weighted concentrations.

## Some results of the sampling campaigns

At all of the three sampling sites, a first flush effect could be observed.

Even at smaller surface loading rates during rain events, none of the plants showed any correlation between traffic intensity and removal efficiencies or between mean surface loading and AFS63 elimination rates.

The described effects are illustrated by the pollutographs shown in Figure 1: Concentrations of AFS63 in inflow and outflow show the high dynamics of pollutants during a rainfall event; Figure 2 shows the observed AFS63 removal efficiencies vs the mean surface loading for a number of different events (one dot per event). The low removal efficiencies, in particular in higher-load scenarios, indicate remobilisation of already separated particles.

Figure 1: Rain event of 01.10.2022 sampled at Paderborn-Gogrevenstraße: Flow and AFS63 concentrations in inflow (“Zu”) and outflow (“Ab”) of rainwater treatment device

Figure 2: Removal efficiencies for AFS63 (η) vs mean surface loading (qA, mittel) for the sampled events (Site: Paderborn-Gogreventraße)

# Modelling

## Description of modelling concept

Figure 3 shows the overall modelling concept: Modelling of AFS (AFS63 and AFScoarse) consists of three submodules (AFS generation – how much AFS is “generated”?; AFS entry – how much AFS is entering the drainage system?; AFS removal – how much AFS is removed in rainwater treatment devices?).

Figure 3: Overall modelling concept: Generation, Entry & Storage, Removal

### AFS generation

The AFS generation module assumes traffic (vehicle tyre and break wear) as one of the sources of AFS. Vehicle numbers (if time series are available from traffic surveys, these can be loaded here by an optional signal input) are multiplied by travelled distance and an AFS load per vehicle (e.g. 0.1 g per vehicle kilometer as frequently found from literature) and with a calibration factor (covering influences such as type of road surface, curvature, frequency of stop-and-move vehicle movements etc.). Additionally, also a constant and/or seasonal AFS generation rate (e.g. seasonal variations by leaves, application of de-icing grit and agents) can be added.

### AFS entry and storage

This module models the internal storage for AFS63 and AFScoarse on the catchment surface, its accumulation and its wash-off. The internal storage capacity Mi (i denoting the pollutant) can be set to finite or infinite values. All AFS63 and AFScoarse entering are assumed to enter the internal storage. The resuspension rate βi determines how much then is leaving from the internal storage, entering the drainage system. Equation 1 denotes the modelling principle, whilst the detailed implementation of the modelling approach is depicted by Figure 4.

| βi := k1,i*qrqr + kq1*MiMi+kM | Equation 1 |
| --- | --- |

with        βi: 	washoff (« resuspension ») rate for pollutant i  [kg/d]

qr             = Max(qin-q1,  0) [m3/d]

qin: 	Inflow (e.g. rainfall runoff) from catchment [m3/d]

q1: 	model parameter [m3/d] (threshold value for flow, at which AFS entry commences)

kq1: 	model parameter [m3/d] (ensuring lower AFS wash-off at low qr)

Mi: 	mass of pollutant i currently in internal storage [kg]

kM: 	model parameter [kg](ensuring lower wash-off when internal storage is near to empty)

Figure 4: Model structure of AFS entry model (in Petersen-Gujer matrix notation; adapted for masses)

### AFS removal

This module determines from the flow, thus from the current values of surface loading, the removal efficiencies according to one of the removal efficiency curves described in the literature (e.g. Fettig et al. (2017), Fuchs and Kemper (2018) [see also A102 guideline] or Stoke’s law). An additional calibration parameter αi covers situations where removal efficiencies are poorer than those indicated by these curves.

## Model application

A model, using the structure described, has been set up for each of the three case studies; for each of them, the rainfall events (altogether, 99 events have been sampled completely) have been split into calibration and validation events and the model calibrated and validated. As calibration criteria for the different modelling modules, the RMSE between measured and simulated AFS63 and AFScoarse loads in inflow and outflow during events have been used. The conference contribution will provide more details on this. As an example, Figure # shows measured and simulated results of the AFS generation module for the set of calibration events for the Paderborn-Bahnhofstraße case study.

Figure #: Cumulated AFS63 loads for calibration events (AFS63 generation module) – Site: Paderborn-Bahnhofstraße

# conclusions and outlook

Application of the model for each of the three case study sites resulted in the observation that one of the main parameters of the AFS entry and storage model (threshold value q1> 0) with a value larger than 0 (thus, different from some earlier modelling approaches) seems to be rather independent from the case study site when expressed by area (with a value of around 0.6 mm/h), whilst some of the other parameters depend more on case-study characteristics. On the other hand, the parameters kM and kq1 can be considered as Monod constants, thus also assumed to be independent of the case-study site. Overall, a promising model approach has been developed and tested against an large sample set obtained by measurement campaigns covering altogether 99 events. Integration of the modelling modules with modules for rainfall-runoff modelling of subcatchments would be a logical next step forward.

# Acknowledgements

Financial support for this project provided by the Ministry of Environment, Nature Protection and Transport of the German Federal State of Northrhine-Westphalia is gratefully acknowledged (Grant reference 17-04.02.01-5/2020). Thanks are also expressed to the staff of the Municipal Wastewater companies of the cities of Paderborn and of Warendorf for their assistance.

LIST OF REFERENCES

DWA-BWK (2020): DWA-A 102-2/BWK-A 3-2 - Grundsätze zur Bewirtschaftung und Behandlung von Regenwetterabflüssen zur Einleitung in Oberflächengewässer - Teil 2: Emissionsbezogene Bewertungen und Regelungen - Dezember 2020

Fettig, J., Pick, V., Liebe, H. (2017): Particle separation from road runoff by a decentralised lamella system – laboratory tests and experiences in the field. Wat. Sci. Tech. 75, 9, 2056-2063

Fuchs, S., Kemper, M. (2018): Empfehlungen für Planung und Betrieb klärtechnischer Maßnahmen. 3. Expertenforum Regenüberlaufbecken RÜB-BW. DWA Regional Chapter Baden-Württemberg, Stuttgart

Rossman, L., Simon, M. (2022): Storm Water Management Model; User's Manual Version 5.2; U.S. Environment Protection Agency

Vinck, E., De Bock, B., Wambecq, Lieskens, E., Delgado, R. (2023): A New Decision Support Tool for Evaluating the Impact of Stormwater Management Systems on Urban Runoff Pollution, Water, 15, 931. https://doi.org/10.3390/w15050931