Global sensitivity analysis for calibrating hydrodynamic sewer models

Analyse de sensibilité globale pour l'étalonnage des modèles hydrodynamiques d'égouts

Matthew W. McGauley and Max Maurer

Swiss Federal Institute for Aquatic Science and Technology (Eawag) and Institute for Environmental Engineering (IfU), ETH Zürich; matthew.mcgauley@eawag.ch, max.maurer@eawag.ch

# RÉSUMÉ

L'eau potable est menacée par les déversements d'eaux usées unitaires (EUU). Les ingénieurs élaborent des modèles hydrodynamiques des bassins versants (par exemple, le modèle de gestion des eaux pluviales, SWMM) afin de quantifier la fréquence et l'ampleur de ces déversements. L'un des défis liés à l'élaboration de ces modèles réside dans leur calibration adéquate, afin que leurs résultats reflètent fidèlement les conditions réelles. Des capteurs (mesurant par exemple le débit ou la profondeur de l'eau) sont généralement installés dans le réseau d'assainissement pour corroborer les résultats du modèle par des mesures in situ. Ce travail propose une méthode permettant de déterminer l'emplacement optimal pour l'installation de ces capteurs, de sorte que les mesures réelles apportent un gain d'information maximal sur le fonctionnement du réseau d'assainissement. Deux analyses de sensibilité globales ont été réalisées sur un modèle de bassin versant dans la banlieue de Zurich, en Suisse, afin de déterminer quels exutoires du modèle présentaient la plus grande variation de volume total de déversement lorsque les bassins versants en amont étaient perturbés (modification de la couverture terrestre, du débit par temps sec et des paramètres d'infiltration). Les résultats préliminaires indiquent que le pourcentage le plus élevé de l'incertitude dans l'ensemble du modèle, qui représente une approximation du gain d'information potentiel lié à l'installation de capteurs, peut être attribué aux seules zones de drainage des égouts unitaires.

# ABSTRACT

Clean, fresh water is threatened by combined sewer overflows (CSOs). Engineers develop hydrodynamic models of sewer catchments (e.g., Stormwater Management Model, SWMM) to quantify the frequency and magnitude of CSOs. A challenge to the development of these models is adequate calibration, such that the model outputs accurately reflect real world conditions. Sensors (e.g., those that measure water flow or depth) are typically installed in the sewer network to corroborate model outputs with in-situ measurements.  This work develops a method to aid in determining the most-impactful location for the installation of such sensors, such that real world measurements result in the largest information-gain about the function of the sewer network. Two global sensitivity analyses were performed on a sewer catchment model in a suburb of Zürich, Switzerland, aimed at determining which outfalls in the model had the largest change in total outflow volume when the upstream catchments were perturbed (i.e., land cover, dry weather flow, and infiltration parameters changed). Preliminary results indicated that the largest percentage of the uncertainty in the entire model, a proxy for potential information gain from installing sensors, could be attributed to the drainage areas of a combined sewer alone.

# KEYWORDS

Combined Sewer Overflows, Hydrodynamic Models, Sensitivity Analysis.

# Introduction

Engineers model combined sewers to understand the frequency and magnitude of harmful Combined Sewer Overflows (CSOs), as well as build mitigation strategies to reduce their occurrence (Riechel et al. 2016). Previous studies have modeled sewersheds to assess the impact that implementing blue-green infrastructure could have on reducing CSOs (Joshi et al. 2021; Rodriguez et al. 2024), which relied heavily on observations from the sewer network to assure that the modeled scenarios were transferable and represented real-world conditions (Blumensaat et al. 2025). Determining the optimal monitoring locations can be challenging due to budget and personnel constraints. Previous studies have noted that sewer networks are comprised of collections of subsystems, where the influence of specific elements can either be immediate or global (Meijer et al. 2018, 2023). Global sensitivity analyses, which are aimed to identify the individual and combined effects of model parameters on a model’s overall uncertainty, could present a complimentary approach to defining the spatial importance of sewershed assets. It was hypothesized that a sequential global sensitivity analysis, where a Morris analysis would first isolate sensitive parameters and a Sobol analysis would use these parameters to quantify the uncertainty contributions from specific drainage zones, would provide a succinct methodology for identifying optimal monitoring locations for maximum information in the model.

# site and methods

## Schaeflibach Rain Basin Catchment

The site studied herein resides in a sewer catchment draining to a rainwater basin in the city of Dietikon, Switzerland, a suburb of Zürich. Three outfalls (8744.00, 4942.00A, and 7503.01 in Figure 1) in the system discharged rainwater to the Schaeflibach river. An outfall of the combined sewer system (A9862.01 in Figure 1) discharged to the Schaeflibach under high-flow conditions but otherwise drained to the main sewer canal (VK_1032 in Figure 1)  that is conveyed to a wastewater treatment plant. A Stormwater Management Model (SWMM) for the catchment was developed (Figure 1). The total modeled area covers 38 hectares across 209 subcatchments. The subcatchments were assigned to unique drainage zones, tracing to the subcatchment’s terminal outfall node. In the case of combined sewer systems, combined, multi-outfall zones were additionally created. This was performed such that the uncertainty attributed to a single subcatchment was not attributed to any single outfall in the combined system. A six-month period of rainfall spanning January to July 2017 was simulated.

Figure 1 Map of the Schaeflibach rain basin catchment area. Colored “X” markers indicate the outfalls in the model. Subcatchments have been color for the “Drainage Zone”, tracing to their terminal outfall.

## Global Sensitivity Analysis

A Morris screening was performed to determine the subcatchment (catchment width, slope, impervious), depression storage (pervious/impervious depression storage depth and percentage with no depression storage), dry weather (inflow rate), and infiltration (max/min infiltration rate and decay rate constant) parameters, which when perturbed, resulted in the largest change in the total discharge volume from the sewer model at the outfalls. Each of the six drainage zones (D), had 10 parameters of interest (P) which would be altered across 21 experimental runs (trajectories, N), where the value of the parameter was set to between 80 and 120% of the original value at random. In all, 1281 scenarios (N * (D*P + 1)) were generated. The total outflow from the sewershed was then aggregated from each simulation and used to determine each parameter’s total sensitivity in the model (mean absolute effect, μ*) as well a confidence interval for the effect. A list of all parameters which significantly contribute to the model’s uncertainty was aggregated, excluding those with a confidence interval overlapping zero, where it could not be determined the parameter had no effect.

A Sobol analysis was then performed aimed at quantifying the exact amount of uncertainty that could be attributed to the individual parameters in the model, accounting for interactions. The analysis was performed in the same spatial organization of drainage zones. This resulted 29 significant parameters from the Morris analysis (P), which would be altered across 200 experimental runs (trajectories, N). In all, 6200 scenarios (N * (P + 2)) were generated at random. The total outflow volume from the sewershed was used to calculate the first-order sensitivity (uncertainty of each parameter alone, S1) and total sensitivity (uncertainty of each parameter, accounting for interactions, ST) of each parameter retained in the analysis. The total sensitivity of each drainage zone in the model was then calculated as the sum of the total sensitivity for each parameter (ST).

# preliminary results and discussion

Among the 10 most-influential parameters to the model’s uncertainty identified in the Morris analysis, the impervious cover of the subcatchments draining to the combined sewer system (A9862.01 & VK_1032 in Figure 1) was found to have the largest effect (Figure 2A.). The impervious cover of the subcatchments draining to the outfalls of the stormwater sewer (4942.00A and A7503.01) as well as the main sewer canal (VK_1032) had an effect that was an order of magnitude less than that of the combined system (Figure 2A.). The combined sewer system, which comprised of 81% of the total modeled area, disproportionately accounted for 99% of the uncertainty in the model’s outflow volume. The remaining zones (comprising of 19% of the total area) contributed less than 1% of the total uncertainty (Figure 2B.).

Figure 2 A. Results of the Morris screening, identifying the 10 most-influential parameters (by zone and model parameter) on the model’s uncertainty, and B. portion of the model’s total uncertainty that can be attributed to each of the drainage zones, as identified by the Sobol analysis that focused on the parameters contributing to the model’s uncertainty.

Increasing the impervious cover of the combined sewershed (Zone A9862.01 and VK_1032) magnified the peak flow out of the outfall for the emergency outfall (A9862.01, Figure 3A.) but had no effect on the peak flow of water to the main sewer trunk (VK_1032, Figure 3A.). At the same time, the total volume of water exiting via these outfalls both increased (Figure 3B.). In effect, the increase in the volume of water exiting via these nodes is both driven by the increase in imperviousness, but via differing mechanisms. For the main sewer trunk, the difference was driven by an increase in the unit hydrograph for all rainfalls. For the emergency outfall, the difference was driven by the magnification of runoff from infrequent extreme events, which remained constant in frequency with respect to imperviousness, as their occurrence was determined by overflow-causing rainfalls. While the imperviousness of other zones was found to contribute to the uncertainty in the model (Figure 2A.), their impact on the total outflow volume from the system was less than that of the combined system (Figure 2B.), despite having been caused by the same mechanism.

Figure 3 A. Peak flow and B. total volume out of the system at the outfalls in the combined sewershed (A9862.01 in blue and VK_1032 in red).

# future work

Future work will extend this analysis to additional catchments and assess adequate locations for the installation of sensors, as well as incorporate groundwater flow and evaporation as inputs. Efforts will focus on installing a flow sensor at the emergency overflow for the combined system, where the largest uncertainty in the model could be attributed to. Upon adequate collection of field data, the observations will be used to calibrate the hydrodynamic models. A pre- and post-calibration assessment will be determined using collected field data, which will facilitate quantifying the true information gain from the sensitivity analyses performed herein. The results of this analysis suggest that it would be imperative to calibrate the impervious cover in the combined sewershed.

# LIST OF REFERENCES

Blumensaat, F., S. Bloem, C. Ebi, A. Disch, C. Förster, M. Maurer, M. Rodriguez, and J. Rieckermann. 2025. “The UWO dataset &ndash; long-term observations from a full-scale field laboratory to better understand urban hydrology at small spatio-temporal scales.” Earth System Science Data Discussions, 1–31. Copernicus GmbH. https://doi.org/10.5194/essd-2024-47.

Joshi, P., J. P. Leitão, M. Maurer, and P. M. Bach. 2021. “Not all SuDS are created equal: Impact of different approaches on combined sewer overflows.” Water Research, 191: 116780. https://doi.org/10.1016/j.watres.2020.116780.

Meijer, D., H. Korving, J. Langeveld, and F. Clemens-Meyer. 2023. “A method to identify the weakest link in urban drainage systems.” Water Science and Technology, 87 (5): 1273–1293. https://doi.org/10.2166/wst.2023.057.

Meijer, D., M. Van Bijnen, J. Langeveld, H. Korving, J. Post, and F. Clemens. 2018. “Identifying Critical Elements in Sewer Networks Using Graph-Theory.” Water, 10 (2): 136. Multidisciplinary Digital Publishing Institute. https://doi.org/10.3390/w10020136.

Riechel, M., A. Matzinger, E. Pawlowsky-Reusing, H. Sonnenberg, M. Uldack, B. Heinzmann, N. Caradot, D. von Seggern, and P. Rouault. 2016. “Impacts of combined sewer overflows on a large urban river – Understanding the effect of different management strategies.” Water Research, 105: 264–273. https://doi.org/10.1016/j.watres.2016.08.017.

Rodriguez, M., G. B. Cavadini, and L. M. Cook. 2024. “Do baseline assumptions alter the efficacy of green stormwater infrastructure to reduce combined sewer overflows?” Water Research, 253: 121284. https://doi.org/10.1016/j.watres.2024.121284.