Exploring the Benefits of Acoustic Signal Monitoring and Machine Learning for Predictive Maintenance in Storm and Wastewater Treatment Systems

Exploration des avantages de la surveillance des signaux acoustiques et de l'apprentissage automatique pour la maintenance prédictive dans les systèmes de traitement des eaux pluviales et usées

Pourya Jafarzadeh1, Gavin Tabor1, Daniel Jarman2 and James L. Webber1

- CWS, University of Exeter, Exeter, UK

{Pj358, G.R.Tabor, J.Webber2}@exeter.ac.uk

- Hydro International Ltd, Clevedon,UK

{djarman}@hydro-int.com

# RÉSUMÉ

Le fonctionnement sûr et fiable des machines est essentiel dans de nombreux secteurs, notamment les transports, l’exploitation minière, l’aérospatiale, la fabrication et le traitement de l’eau. La détection de défauts joue un rôle central dans la surveillance d’état, facilitant le diagnostic, le pronostic et une planification optimale de la maintenance pour réduire les coûts et prolonger la durée de vie des équipements. Avec les progrès de l’IoT et des capteurs, la disponibilité des données a fortement augmenté, stimulant l’usage de l’apprentissage automatique pour la surveillance d’état. Cependant, ces approches demeurent peu explorées dans le traitement des eaux usées. Cette recherche vise à étudier l’application de méthodes d’apprentissage automatique à la surveillance d’état des équipements de traitement des eaux usées. Un modèle à transformeurs, BEATs, a été appliqué à un jeu de données acoustiques public via une approche de détection d’anomalies. Les résultats montrent un taux de vrais positifs élevé, un faible taux de faux positifs et une AUC moyenne de 95,22 % en présence de bruit réel d’usine. Ces performances démontrent le potentiel du suivi acoustique et des modèles à transformeurs pour une surveillance d’état fiable dans les systèmes de traitement des eaux usées.

# ABSTRACT

The safe and reliable operation of machinery is essential in many industries, including transportation, mining, aerospace, manufacturing, and water treatment. Fault detection is a key component of machine condition monitoring, guiding decisions on resource allocation for diagnosis and prognosis, and enabling effective maintenance scheduling to reduce costs, enhance productivity, and extend equipment life. With advances in IoT and sensor technologies, data availability has grown significantly, leading to widespread use of machine learning and deep learning for condition monitoring across various sectors. However, despite this progress, these approaches remain underexplored in the wastewater treatment industry. This research therefore aims to investigate machine learning methods for condition monitoring in wastewater treatment devices, an area not previously studied.  In this study, a transformer-based model, BEATs, was applied to a public acoustic dataset using an anomaly detection approach. The experimental results showed that the method achieved a high true positive rate (TPR) and a low false positive rate (FPR), demonstrating the reliability of the system for condition monitoring. The mean AUC of the model was 95.22% for the device with real factory noise. These results highlight the potential of acoustic-based monitoring and transformer models for supporting reliable condition monitoring in wastewater treatment systems.

# KEYWORDS

Acoustic signal, Condition monitoring, Machine learning, Predictive maintenance, Wastewater treatment

# introduction

With the rapid growth of industrial data driven by advances in the Internet of Things (IoT) and sensor technologies, deep learning models have become powerful tools for predictive maintenance (PdM) and condition monitoring (CM). Maintenance strategies can be reactive, preventive, or predictive. Reactive maintenance, also known as corrective or run-to-failure, is the most expensive because it does not use condition monitoring and components are replaced only when defects occur or accumulate. Preventive, or scheduled, maintenance replaces components at planned intervals, ideally before faults appear. Predictive maintenance, supported by CM, identifies components that are likely to fail so they can be replaced on time (Stetco, et al., 2019). These diagnostic techniques improve plant efficiency, reduce replacement costs, and prevent financial losses from unexpected breakdowns. In this paper, we present the conceptual basis of expanding current practice regarding acoustic CM into the water sector.

## The Benefits of Acoustic Signals in Condition Monitoring

Acoustic sensors are non-intrusive and do not require physical contact with equipment. Waterproof microphones can operate in wet environments, and acoustic sensing works in areas where cameras are ineffective. These sensors are also inexpensive and simple to deploy. In contrast, vibration monitoring relies on contact sensors such as accelerometers and proximity probes. These require installation on the machine, which may be restricted by cost, space, or potential impact on the equipment. When contact sensors cannot be used, acoustic sensing provides a flexible alternative. The main limitation is sensitivity to background noise from other machines and to changes in operating conditions. Also, acoustic signals remain suitable for real-time monitoring because they capture information from multiple components simultaneously. They also cover a wide frequency range from 20 Hz to 20 kHz. Many industrial accelerometers operate within a narrower range, often up to about 10 kHz. Acoustic sensing avoids issues such as mass loading and the need for flat mounting surfaces, and data can be recorded even in very wet conditions (Delvecchio, et al., 2018).

These advantages make acoustic sensing suitable for condition monitoring in water and wastewater treatment environments. Waterproof microphones can be used continuously in moist conditions, and mechanical components such as screens generate both low and high frequency content that is well captured by acoustic measurements. The non-intrusive nature of acoustic sensors supports reliable monitoring without disturbing normal operation.

## Condition Monitoring in the Wastewater Treatment Industry

Previous studies have carried out extensive work on predictive maintenance (PdM) and fault detection for many types of equipment using machine learning. These include turbines, rotating machinery such as roller bearings and gearboxes, fans, maritime systems, water treatment processes such as oxidation and nitrification (Mamandipoor, et al., 2020), and pipe leakage detection (Saleem, et al., 2025), (Banjara, et al., 2020). However, there is still a clear gap in applying machine learning to wastewater treatment devices in real industrial settings.

There is a clear need to address this issue, based on the operational and financial consequences of failures in the water sector. It is essential that treatment devices operate continuously. In December 2024, a major technical failure that resulted in a widespread water outage in the UK that affected around 58,000 properties. Nearly £10 million in automatic compensation had to be paid by the company. It is possible to prevent such incidents by installing a real-time monitoring system in treatment plants, to reduce costs, and to increase long-term reliability.

### Selection of Wastewater Treatment Devices for Acoustic-Based Condition Monitoring

Failure of wastewater screening systems allows large debris and solids to reach downstream processes, causing clogging, equipment damage, increased pump and filter wear, and higher maintenance costs. Coarse screens protect pumps, valves, and pipelines from large objects and typically precede fine screens, though they may be unnecessary in some industrial plants.  Also, dissolved air flotation (DAF) clarifies wastewater by forming tiny air bubbles that lift suspended solids, oils, and other impurities to the surface for removal. DAF system failures can reduce effluent quality, overload downstream processes, violate regulations, and disrupt overall treatment performance.

# Proposed Method

The transformer architecture, first introduced by Google for natural language processing (Vaswani, et al., 2017), has become influential in fields such as computer vision and speech processing because of its strong performance. Transformers use attention mechanisms together with positional encoding, encoder–decoder structures, masked attention and feed-forward networks. This allows them to model long-range dependencies and process data in parallel, avoiding the limits of recurrent and convolutional models.

In this study, we have applied the BEATs model (Chen, et al., 2023), because it provides an opportunity for transfer learning from a large pre-trained audio datasets such as AudioSet. This is of significance, as there is currently no acoustic dataset for wastewater and stormwater treatment devices, representing is a clear gap in condition monitoring research. Therefore, for the experiments in this paper, we have applied the MIMII dataset (Purohit, et al., 2019), MIMII contains a large range of devices, including pumps, sliders and fans. These have similar auditory signatures to equipment used in wastewater treatment plants. The dataset also contains real factory environmental background noise mixed with the target machines sounds, adding a degree of realism to our experiment.

In real industrial environments, anomalous sounds are rare and highly varied, so it is not possible to collect complete patterns. This requires detecting unknown anomalies that do not appear in the training data. Normal data is also far more common than abnormal data, and labelling is difficult, while recording faulty behaviour can be costly or unsafe. A practical solution is anomaly detection trained only on normal data using a semi-supervised method. We consider this approach more suitable than classification for identifying early faults in water treatment equipment.

# Experimental Results

In the fine-tuning stage, only normal device sounds are used, while evaluation considers both normal and abnormal sounds. We measured how far each test sample deviated from the distribution of normal data using  a statistical distance measure (Mahalanobis distance). In the test set, the positive class is abnormal data, and the negative class is normal data. Two main metrics for assessing anomaly detection systems are the AUC and the partial AUC (pAUC). The AUC indicates the overall balance between the true positive rate (TPR) and the false positive rate (FPR) across all thresholds. The pAUC (for example, pAUC at FPR ≤ 0.1) evaluates performance in the low-FPR range, where achieving a high TPR is especially crucial to minimise false alarms in real-world scenarios. In Table 1, the average AUC and pAUC of the proposed method are presented and compared with other models on the MIMII dataset across 10 independent tests.

Our model compared with CNN, LSTM, and autoencoder based models, the experimental results show that our approach is trustworthy for condition monitoring of the devices that are close to water sector such as pumps and valves. In the dataset, the sounds of pumps are mixed with water, highlighting the relevance of the selected dataset to the problem. Moreover, the proposed model demonstrated strong performance on both stationary signals, such as fan sounds, and non-stationary signals, including valve sounds, which contain impulsive events. Additionally, we tried to tackle the problem using both rotary and translational motions, such as pumps and sliders, respectively. It is worth noting that the lack of a publicly available, high-quality acoustic dataset for wastewater treatment devices is another tangible gap. To support future research in this area, we are collecting and providing such a dataset.  In the figures, the ROC curves for the four devices are shown, and the pAUC for each is highlighted. This demonstrates the reliability of the system in detecting true positives (real faults or anomalies) while keeping the false positive rate low at 0.1.

Table 1. AUC and pAUC of the proposed method on MIMII dataset.

| Method | Fan (AUC) | Pump (AUC) | Slider (AUC) | Valve (AUC) | Fan (pAUC) | Pump (pAUC) | Slider (pAUC) | Valve (pAUC) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BEATs | 86.70 | 95.68 | 99.16 | 99.55 | 76.05 | 88.04 | 95.57 | 97.73 |
| IDNN/IAE+ MobileNetV2 | 82.33 | 86.94 | 97.28 | 97.38 | 84.30 | 81.73 | 89.73 | 90.89 |
| VAE+CAE+CNN | 92.86 | 92.98 | 98.95 | 97.77 | 83.53 | 87.23 | 94.54 | 94.54 |
| AE | 65.83 | 72.89 | 84.76 | 66.28 | 52.45 | 59.99 | 66.53 | 50.98 |
| CAE | 66.78 | 72.07 | 91.77 | 78.83 | 52.63 | 60.96 | 76.20 | 53.10 |
| LSTM | 67.32 | 73.94 | 84.99 | 67.82 | 52.05 | 61.01 | 67.47 | 51.07 |
| IDC-LSTM-AE | 65.83 | 69.67 | 74.48 | 59.71 | 65.83 | 69.67 | 74.48 | 59.71 |

Figure 1 Fan device with AUC=97.05 and pAUC=88.95.

Figure 2 Pump device with AUC=99.0 and pAUC=96.02.

Figure 3 Slider device with AUC=99.99 and pAUC=99.99.

Figure 4 Valve device with AUC=100 and pAUC=100.

# Bibliography

Banjara, N. K., Sasmal, S. & Voggu, S., 2020. Machine learning supported acoustic emission technique for leakage detection in pipelines. International Journal of Pressure Vessels and Piping, Volume 188, p. 104243.

Chen, S. et al., 2023. BEATs: Audio pre-training with acoustic tokenizers. In: Proceedings of the 40th International Conference on Machine Learning. s.l.:PMLR, p. 5178–5193.

Delvecchio, S., Bonfiglio, P. & Pompoli, F., 2018. Vibro-acoustic condition monitoring of internal combustion engines: A critical review of existing techniques. Mechanical Systems and Signal Processing, Volume 99, p. 661–683.

Mamandipoor, B. et al., 2020. Monitoring and detecting faults in wastewater treatment plants using deep learning. Environmental Monitoring and Assessment, Volume 192, p. 148.

Purohit, H. et al., 2019. MIMII dataset: Sound dataset for malfunctioning industrial machine investigation and inspection. s.l., arXiv.

Saleem, F., Ahmad, Z. & Kim, J.-M., 2025. Real-time pipeline leak detection: A hybrid deep learning approach using acoustic emission signals. Applied Sciences, 15(1), p. 185.

Stetco, A. et al., 2019. Machine learning methods for wind turbine condition monitoring: A review. Renewable Energy, p. 620–635.

Vaswani, A. et al., 2017. Attention is all you need. Advances in Neural Information Processing Systems, Volume 30.